s0 – ready / predpogoji (idle)
interlock 
• safety ok = 1
• air ok = 1
• mag present = 1
• box present = 1
• r1 ready = 1 in r1 fault = 0
• r2 ready = 1 in r2 fault = 0
• main carr (glavni vozek) fault = 0, packing carr (vozek embalaža) fault = 0
• tray present = 1 (na packing carr mora biti prazna embalaža)
• punch up = 1, test up = 1, support down = 1 (varno začetno stanje)
izvedba
• nič (čaka)
alarmi
 • če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low
• če mag1 present = 0 ali mag2 present = 0 → magazine missing (mag1 or mag2 not in place)
• če tray present = 0 → no empty tray on packing carr

s1 – homing 
interlock 
• safety ok = 1
• air ok = 1
• r1 fault = 0 in r2 fault = 0
• main carr fault = 0 in packing carr fault = 0

izvedba
• preveri r1 home = 1 in r2 home = 1

- če r1 home ni 1 → pošlji robotu 1 nalogo: go home, čaka r1 home = 1 (ali r1 fault, timeout)

- če r2 home ni 1 → pošlji robotu 2 nalogo: go home, čaka r2 home = 1 (ali r2 fault, timeout)

• ko sta r1 home = 1 in r2 home = 1 preveri cilindri

- če test up ni 1 → ukaz: test up, čaka test up = 1 (timeout ali alarm)

- če punch up ni 1 → ukaz: punch up, čaka punch up = 1 (timeout ali alarm)

- če support down ni 1 → ukaz: support down, čaka support down = 1 (timeout ali alarm)

• ko so roboti in cilindri v varnem stanju, naredi homing vozičkov

- če main carr homed = 0 → homing main carr (z main carr home l + main carr home r za poravnavo)

- če packing carr homed = 0 → homing packing carr

- preveri main carr lim * in packing carr lim * niso aktivni (če so, stop)

ok pogoj
• r1 home = 1 in r2 home = 1
• test up = 1 in punch up = 1 in support down = 1
• main carr homed = 1 in packing carr homed = 1

timeout ali alarm
• timeout pri robot go home ali r1 fault oziroma r2 fault
• timeout pri vračanju cilindrov v začetno lego
• homing timeout ali main carr fault oziroma packing carr fault
• safety ok = 0 ali air ok = 0 kadarkoli v s1

s2 – sočasno r1 prenese panel v poravnalno šablono
in r2 naloži prazno embalažo na packing carr
interlock 
• safety ok = 1
• air ok = 1
• r1 ready = 1 in r1 fault = 0
• r2 ready = 1 in r2 fault = 0
• main carr v load poziciji = 1 (to je home za main carr)
• packing carr v poziciji za nalaganje embalaže = 1 (to je home za packing carr)
• test up = 1
• punch up = 1
• support down = 1
• box empty present = 1 (škatla s prazno embalažo je na mestu)
• mag1 present = 1 in mag2 present = 1

izvedba
• zažene dve nalogi hkrati (parallel)

a) r2 naloga: naloži prazno embalažo na packing carr
• r2 cmd: pick empty tray from box empty
• med prijemom: preverjaj r2 vacuum ok = 1

- če r2 vacuum ok ni 1 v definiranem času → alarm: no vacuum on r2 gripper
• r2 cmd: place empty tray on packing carr
• po odložitvi preveri tray present na packing carr = 1 (foto senzor pod packing carr)

- če tray present ni 1 → alarm: tray not detected on packing carr
• r2 done = 1

b) r1 naloga: vzemi panel iz magazina in ga položi na poravnalno šablono
• r1 cmd: pick panel from magazine with gripper force = nastavljen force
• po zapiranju gripperja preveri r1 gripper force closed = as specified

- če r1 gripper force closed < specified → alarm: no grip force, panel not gripped
• r1 cmd: place panel in jig
• po odložitvi preveri jig panel present = 1 (foto senzor v šabloni)

- če jig panel present ni 1 → alarm: panel not detected in jig
• r1 done = 1

ok pogoj
• r2 done = 1 in tray present na packing carr = 1
• r1 done = 1 in jig panel present = 1
• brez r1 fault in brez r2 fault

timeout ali alarm
• r2 timeout ali r2 fault
• r2 vacuum ni ok → no vacuum on r2 gripper
• tray present na packing carr → tray not detected on packing carr
• r1 timeout ali r1 fault
• r1 gripper force closed < specified → no grip force, panel not gripped
• jig panel present → panel not detected in jig
• safety ok = 0 ali air ok = 0 kadarkoli v s2

s3 – r1 vakuum preprijem, odlaganje na main carr, hkrati packing carr v pozicijo pod štanco

interlock 
• safety ok = 1
• air ok = 1
• r1 ready = 1 in r1 fault = 0
• main carr v load poziciji = 1
• jig panel present = 1 (panel je v poravnalni šabloni)
• test up = 1
• punch up = 1
• support down = 1
• packing carr fault = 0
• tray present na packing carr = 1 (prazna embalaža je naložena)

izvedba
• zaženi dve nalogi hkrati (parallel)

a) r1 naloga: vakuum preprijem panela iz šablone, odlaganje na main carr, nato r1 home
• r1 cmd: vac pick panel from jig
• preveri r1 vacuum ok = 1 v definiranem času

- če r1 vacuum ok = 0 → no vacuum on r1 vacuum gripper
• r1 cmd: place panel on main carr
• po odložitvi preveri panel on main carr = 1 (foto)

- če panel on main carr = 0 → panel not detected on main carr
• priporočeno: po odložitvi preveri r1 vacuum ok = 0 (da je panel spuščen)

- če r1 vacuum ok = 1 → vacuum not released
• r1 cmd: go home
• preveri r1 home = 1

- če r1 home = 0 → r1 not in home position
• r1 done = 1

b) packing carr naloga: premik v pozicijo pod štanco
• ukaz pogonu: packing carr → in punch
• potrditev: packing carr pos in punch = 1
• potrditev: tray in punch zone = 1 (foto)
• potrditev: packing carr under punch = 1 (foto senzor)

ok pogoj
• r1 done = 1
• panel on main carr = 1
• r1 home = 1
• packing carr pos in punch = 1
• tray in punch zone = 1
• packing carr under punch = 1
• brez r1 fault in brez packing carr fault

timeout ali alarm
• r1 timeout ali r1 fault
• če r1 vacuum ok = 0 → no vacuum on r1 vacuum gripper
• če panel on main carr = 0 → panel not detected on main carr
• če r1 vacuum ok = 1 po odložitvi → vacuum not released
• če r1 home = 0 → r1 not in home position
• packing carr timeout ali packing carr fault
• če tray in punch zone = 0 → tray not in punch zone
• če packing carr under punch = 0 → packing carr not detected under punch
• safety ok = 0 ali air ok = 0 kadarkoli v s3

s4 – main carr premik na pozicijo testiranje

interlock
• safety ok = 1
• air ok = 1
• main carr fault = 0
• panel on main carr = 1
• r1 home = 1
• test up = 1
• punch up = 1
• support down = 1

izvedba
• ukaz pogonu: main carr → test

ok pogoj
• main carr pos test = 1

timeout ali alarm
• timeout pri premiku main carr v test pozicijo
• main carr fault = 1
• če test up = 0 → test head not in safe position
• če punch up = 0 → punch not in safe position
• če support down = 0 → support not in safe position

s5 – podpora gor

interlock 
• safety ok = 1
• air ok = 1
• main carr pos test = 1
• test up = 1
• punch up = 1
• support down = 1

izvedba
• vklopi ventil: support up
• čaka support up = 1

ok pogoj
• support up = 1

timeout ali alarm
• timeout pri dvigu podpore
• če support up = 0 → support not in up position
• če air ok = 0 → air pressure low
• če safety ok = 0 → safety not ok

### s6 – testiranje (cylinder b, test down, test start, feedback, test up)

interlock 
• safety ok = 1
• air ok = 1
• main carr pos test = 1
• support up = 1
• punch up = 1
• r1 home = 1
• r2 home = 1

izvedba
• vklopi ventil: test down
• čaka test down = 1
• pošlji testerju: test start = 1
• čaka test done = 1
• preberi feedback: test pass ali test fail (ali test result)
• vklopi ventil: test up
• čaka test up = 1

ok pogoj
• test down = 1
• test done = 1
• test pass = 1 ali test fail = 1
• test up = 1

timeout ali alarm
• timeout pri test down
• če test down = 0 → test head not in down position
• timeout pri test done
• če test done = 0 → test not completed
• če test pass = 0 in test fail = 0 → test result missing
• timeout pri test up
• če test up = 0 → test head not in up position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

### s7 – spust podpore (cilinder a)

interlock 
• safety ok = 1
• air ok = 1
• main carr pos test = 1
• test up = 1
• punch up = 1
• support up = 1

izvedba
• vklopi ventil: support down
• čaka support down = 1

ok pogoj
• support down = 1

timeout ali alarm
• timeout pri spustu podpore
• če support down = 0 → support not in down position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

### s8 – premik main carr v pozicijo za štanco

intelock
• safety ok = 1
• air ok = 1
• main carr fault = 0
• main carr pos test = 1
• panel on main carr = 1
• r1 home = 1
• test up = 1
• punch up = 1
• support down = 1

izvedba
• ukaz pogonu: main carr → punch (pozicija za štanco)

ok pogoj
• main carr pos punch = 1

timeout ali alarm
• timeout pri premiku main carr v punch pozicijo
• main carr fault = 1
• če test up = 0 → test head not in safe position
• če punch up = 0 → punch not in safe position
• če support down = 0 → support not in safe position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

### s9 – izsek iz panela (štanca dol, cilinder c)

intelock
• safety ok = 1
• air ok = 1
• main carr fault = 0
• packing carr fault = 0
• main carr pos punch = 1
• main carr under punch = 1
• packing carr pos in punch = 1
• tray in punch zone = 1
• punch up = 1
• test up = 1
• support down = 1
• r1 home = 1
• r2 home = 1

izvedba
• vklopi ventil: punch down
• čaka punch down = 1

ok pogoj
• punch down = 1

timeout ali alarm
• timeout pri punch down
• če punch down = 0 → punch not in down position
• če main carr under punch = 0 → main carr not detected under punch
• če tray in punch zone = 0 → tray not in punch zone
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

s10 – izsek iz panela (štanca gor, cilinder c)

intelock
• safety ok = 1
• air ok = 1
• punch down = 1

izvedba
• vklopi ventil: punch up
• čaka punch up = 1

ok pogoj
• punch up = 1

timeout ali alarm
• timeout pri punch up
• če punch up = 0 → punch not in up position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

### s11 – hkratni povratek vozičkov na home (main carr home, packing carr home)

intelock
• safety ok = 1
• air ok = 1
• main carr fault = 0
• packing carr fault = 0
• punch up = 1
• test up = 1
• support down = 1

izvedba
• zaženi dve nalogi hkrati (parallel)

- ukaz pogonu: main carr → home (home = load pozicija)

- ukaz pogonu: packing carr → home (home = pozicija za nalaganje in prevzem embalaže)

ok pogoj
• main carr pos home = 1
• packing carr pos homs12 enaka logika za r2e = 1

timeout ali alarm
• timeout pri premiku main carr v home pozicijo
• timeout pri premiku packing carr v home pozicijo
• main carr fault = 1
• packing carr fault = 1
• če punch up = 0 → punch not in safe position
• če test up = 0 → test head not in safe position
• če support down = 0 → support not in safe position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

s12 – dvig in umik v safe pozicijo (r1 in r2)

intelock
• safety ok = 1
• air ok = 1
• r1 ready = 1 in r1 fault = 0
• r2 ready = 1 in r2 fault = 0
• main carr pos home = 1
• packing carr pos home = 1
• test up = 1
• punch up = 1
• support down = 1

izvedba
• zaženi dve nalogi hkrati (parallel)

a) r1 naloga: pobere prazni panel in gre v safe pozicijo nad main carr
• r1 cmd: vac pick empty panel from main carr
• preveri r1 vacuum ok = 1

- če r1 vacuum ok = 0 → no vacuum on r1 vacuum gripper
• r1 cmd: move to r1 safe above main carr
• preveri r1 safe = 1

- če r1 safe = 0 → r1 not in safe position
• r1 done = 1

b) r2 naloga: pobere polno embalažo in gre v safe pozicijo nad packing carr
• r2 cmd: vac pick full tray from packing carr
• preveri r2 vacuum ok = 1

- če r2 vacuum ok = 0 → no vacuum on r2 vacuum gripper
• r2 cmd: move to r2 safe above packing carr
• preveri r2 safe = 1

- če r2 safe = 0 → r2 not in safe position
• r2 done = 1

ok pogoj
• r1 done = 1 in r1 safe = 1 in r1 vacuum ok = 1
• r2 done = 1 in r2 safe = 1 in r2 vacuum ok = 1

timeout ali alarm
• r1 timeout ali r1 fault
• če r1 vacuum ok = 0 → no vacuum on r1 vacuum gripper
• če r1 safe = 0 → r1 not in safe position
• r2 timeout ali r2 fault
• če r2 vacuum ok = 0 → no vacuum on r2 vacuum gripper
• če r2 safe = 0 → r2 not in safe position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low

s13 – odlaganje v škatle in sprostitev vakuuma, nato home (r1 in r2)

intelock
• safety ok = 1
• air ok = 1
• r1 safe = 1
• r2 safe = 1
• r1 fault = 0
• r2 fault = 0

izvedba
• zaženi dve nalogi hkrati (parallel)

a) r1 naloga: odloži prazni panel v box za prazne panele, spusti vakuum, home
• r1 cmd: place panel in empty panel box
• ukaz: vacuum off
• čaka r1 vacuum ok = 0

- če r1 vacuum ok = 1 → vacuum not released
• r1 cmd: go home
• preveri r1 home = 1

- če r1 home = 0 → r1 not in home position
• r1 done = 1

b) r2 naloga: odloži polno embalažo v škatlo za polno embalažo, spusti vakuum, home
• r2 cmd: place full tray in full tray box
• ukaz: vacuum off
• čaka r2 vacuum ok = 0

- če r2 vacuum ok = 1 → vacuum not released
• r2 cmd: go home
• preveri r2 home = 1

- če r2 home = 0 → r2 not in home position
• r2 done = 1

ok pogoj
• r1 done = 1 in r1 home = 1 in r1 vacuum ok = 0
• r2 done = 1 in r2 home = 1 in r2 vacuum ok = 0

timeout ali alarm
• r1 timeout ali r1 fault
• če r1 vacuum ok = 1 po vacuum off → vacuum not released
• če r1 home = 0 → r1 not in home position
• r2 timeout ali r2 fault
• če r2 vacuum ok = 1 po vacuum off → vacuum not released
• če r2 home = 0 → r2 not in home position
• če safety ok = 0 → safety not ok
• če air ok = 0 → air pressure low